North Dakota Fumigation Management Plan

Farm Operations

This plan is developed to help farm operations comply with fumigation regulations established by the state of North Dakota. It does not address every possible fumigation-related situation.  Any relevant aspect of the operation specific to the site this plan is designed for must be included with these plans.

Developed for:

Your name:   	      ______________________________________________

Address:	      ______________________________________________

______________________________________________

City, ST, zip:	      ______________________________________________

Owner/Responsible Manager

Name:_____________________________________

Day phone: _________________________________

Night phone: ________________________________

Cell phone: __________________________________

Pager:  ______________________________________

Email: ______________________________________

Certified Applicator

Name: ___________________________________

Certification no. ___________________________

Exp. Date: _________________________

Day phone: _______________________________

Night phone: _____________________________

Cell phone: _______________________________

Pager: ___________________________________

Emergency Information

Local Police: _____________________________________________ (or 911)

Local Fire: _______________________________________________ (or 911)

Local ER:_____ ___________________________________________

Local Ambulance: ________________________________________ (or 911)

County Sheriff: ___________________________________________

County Emergency Management: ___________________________

ND Dept. of Ag 	701-328-2231

Chemtrec:				1-800-424-9300

Poison Control:						1-800-222-1222

FMP Checklist for Compliance

A.	Preparation and Planning

1.	Read and review relevant literature and product information:

  - Is another Fumigation Management Plan in place?

- No

- Yes, reviewed:		Date _______________________

- MSDS (Material Safety Data Sheet)

- Product label and applicator (or product) manual

- Reviewed:  Date _______________________

- Other relevant material

- None

- Respiratory equipment instruction

  - reviewed:       Date ___________________

- Gas detection equipment instruction

  - reviewed:       Date ___________________

- SCBA equipment instruction, if applicable

  - reviewed:       Date ___________________

2.	Obtain or develop a drawing of the layout of the structure to be fumigated, and attach it to this FMP document, to be used to verify measurements and to make site-specific notes as part of the FMP. If the following notations do not exist on the plan, make note of the location of:

  - Drains

    - Doors, windows,  vents, pipe through-passes, and entryways that will need to be    sealed

  - Doors and entryways that will need to be secured to prevent entry

    - Connections (including electrical) to adjacent areas and other buildings not scheduled for fumigation

    - Areas where new construction or remodeling have occurred since the last fumigation

  - Fans and the switches and breakers that control them

  - Location of water sources

- Inspect the structure for its suitability for fumigation, and consult with previous FMPs and other records for any changes to the structure. (Examples that would not meet suitability criteria: wooden or other construction that can’t be adequately sealed, too close to residential buildings or other inhabited areas, etc)

- Structure suitable for fumigation

- Air handling equipment that will be shut down during the treatment and utilized during aeration has been reviewed.

  - Switches marked on the facility drawing, and flagged or marked for easy identification during the aeration process.

- Changes to structure from previous FMP

- No

- Yes, changes noted on FMP Site Drawing, Appendix 3

- The person responsible for the fumigation will be:

- licensed in the Fumigation category through the NDSU Pesticide Program, under the North Dakota Department of Agriculture

- physically present during the application of the fumigant and for the initial opening of the fumigated structure.

- Other fumigation personnel will be trained in fumigation procedures

- Specialized training is documented in employees file.

Name _____________________________

- Certified fumigation applicator

- Trained applicator

- Gas detector training

- SCBA training

- Medically approved as per OSHA

- Fit tested

- Respiratory equipment training

Name _____________________________

- Certified fumigation applicator

- Trained applicator

- Gas detector training

- SCBA training

- Medically approved as per OSHA

- Fit tested

- Respiratory equipment training

- Personal Safety Equipment for fumigators

  - Safety equipment is owned by this farm and has been properly maintained, and serviced according to manufacturer requirements (check only those used)

    - Dry, cotton gloves

    - Individual respirators

    - Gas detectors

    - SCBA units

- Respirators, gas detectors, or SCBA units are not owned by this farm, but supplied by:

- Respirator  supplier: __________________________________

- Gas Detector supplier: _________________________________

- SCBA supplier: _______________________________________

- Proper maintenance and service of borrowed/rented equipment has been verified

- Safety of People not involved in fumigation

  - If the structure to be fumigated is attached to, adjacent to, or somehow connected or within another structure not scheduled for fumigation, the following steps will be taken to ensure there will be no unsafe exposure to fumigant:

- Connections will be sealed off using polyethylene sheeting, tape

- Gas readings will be taken in adjacent and/or connected areas at intervals of ______hours while adjacent areas are occupied, and recorded on Appendix 2

- Continuous electronic monitoring of gas levels will be conducted by means of (specific device):  _________________________________________________

- Supplying workers with gas-detecting devices

Other   _______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Emergency Response Procedures

- Those involved in this fumigation have been informed of the procedural steps listed

  - Evacuate the immediate area of the release of fumigant

  - Monitor gas concentrations and put on respiratory equipment as needed

  - Cordon off area, approximately 100 feet around the point of release, with “Danger” tape and attach fumigant placards to tape

  - Report any accident and/or incidents related to fumigant exposure to:

________________________________________________

- Submit the “Official Notification of Fumigation” (see Appendix 1) to the Fire Chief twenty four hours prior to the fumigation, unless otherwise notified by the fire department. Record the signature of the Fire Chief/Authorized Personnel on this form.

B.	Fumigation Procedures

- What is the commodity to be fumigated?

- Raw agricultural product

- Feed

Condition of commodity:

  - Moldy

  - Infested, if so, what pests  _____________________________________________________

Volume of commodity:

- _______________________ cu ft, or

- _______________________ bu

Previous treatment of commodity:

  - No

  - Product ___________________________________________

  - Date ___________________________________

2.	Exposure time calculations

Fumigant to be used: ___________________________________________________________

Commodity (or empty bin) temperature:  ________________ 0 F  or _______________0 C

Minimum fumigant exposure at measured temperature: ____________________ hours.

Fumigation start: day _____________, date _______________, time ____________

Anticipated fumigation end:  day___________, date_____________, time__________

Total down time (incl. time for sealing, fumigant exposure, aeration, and testing):

Day _____, date ________, time _______     until day_______, date_______, time ________

Commodity moisture:   ____________________%

Deactivation method planned:  __________________________________________________

______________________________________________________________________________

3.	Dosage considerations:   (note: refer to product manual or label for appropriate range of dosage)

Area to be fumigated:  ___________________cu ft   or  _____________________bu.

Labeled rate: _____________________/1000 cu ft   or   _____________________/1000 bu.

Amount of product required by label or manual:

______ pellets

______ flasks of 1660 pellets each

______ tablet prepacs

______ Magtoxin prepacs

______ Fumi-cels

______ Fumi-strips

______ pounds of Eco2Fume®

_______ other ____________________

- Sealing Procedures

- Mark on FMP, all equipment or locations within the area to be fumigated that must be sealed off or isolated. Sophisticated electronic equipment (panel boards, computers) must be isolated from phosphine gas to prevent potential corrosion damage.

- If the site has been fumigated before, review the previous FMP for sealing information.

  - No structural changes that affect sealing procedures have occurred as the result of construction/remodeling

  - Structural changes (i.e. construction/remodeling) ,that affect sealing, have been noted and marked on FMP

- Prepare the structure for the fumigation and complete the required sealing.

- Placarding and Locking

- Placard contains the following information:

  - “Danger/Peligro”

  - Skull and crossbones

  - Date and time fumigation begins and ends

  - Fumigant product name and EPA registration

  - Name, Address, 24-hour phone number for applicator

- Place placards on doors, hatches, and ladders (mark with ‘P’ on FMP drawing)

- Other (describe)  ____________________________________________________________________________________________________________________________________________________________

- Monitoring - Safety Monitoring

- Detection equipment necessary for monitoring gas levels and clearing fumigated areas is available and calibrated, if necessary.

- Detector tubes have not expired

- Conduct phosphine gas monitoring in areas to prevent excessive exposure and to determine where exposure may occur. Document where monitoring occurred and what the gas concentrations were (see Appendix 2).

- Monitoring for fumigant levels during fumigation documented, see form Appendix 2

- Fumigant levels will be monitored using

- ___________________________________________________________

(identify gas detection device)

- at intervals of _____________________hrs

- Check for possible leaks near the treatment site and any other critical areas around or near the fumigation site such as areas downwind from the site.

  - None detected

  - Leak locations, and corrections, noted on drawing and in Appendix 3.

Monitoring - Efficacy Monitoring

- Take gas readings from within the fumigated structure to ensure proper gas concentrations.  Record on Appendix 2.

C.	Post-Application Operations

1.	Check fumigant concentration with a gas detector before re-entry

- Fumigant level:  ___________________________

- Opening and Aerating Fumigated site

  - Aeration scheduled to begin at:

    - Day __________________________

    - Date __________________________

    - Time __________________________ (A.M or P.M)

  - Documentation of gas levels during aeration recorded on Appendix 2

- Deactivation of spent fumigant

  - Dry

  - Wet

- Disposal of deactivated spent fumigant

____________________________________________________________________________________________________________________________________________________________

(describe method)

- Count all spent phosphine products to ensure that all fumigant used on the job has been retrieved.

- Plan for additional cleanup and preparation of site for commodity release:

- None necessary

- Other __________________________________________________________________

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Remove warning placards when aeration is complete.

- Notify farm management that personnel, contractors or others may be allowed to re-enter the aerated area.

Appendix 1:

OFFICIAL NOTICE of FUMIGATION

Farm name	___________________________________________

Address: 		___________________________________________

___________________________________________

___________________________________________

Pesticide being used: 	_____________________________________

Date of application:	_____________________________________

Date of aeration:		_____________________________________

Certified Applicator in charge:	_____________________________________

(print name)

_____________________________________

(sign name)

_____________________________________

(date)

Phone #	_____________________

Cell #		_____________________

Pager #	_____________________

Fire Chief/Authorized Official	_________________________________________

(Signature)

__________________________________

(date)

Notice to Fire Chief or Authorized Official:

Please return this notification via fax to the elevator at: _____________________

(fax number)

Appendix 2:

PHOSPHINE FUMIGATION GAS MONITORING REPORT

Business name:  _________________________________________________

Date:		   ________________________

Efficacy Gas Concentration Monitoring

| Location: | Date: ____ Time: ____ Initial: ____ | Date: ____ Time: ____ Initial: ____ | Date: ____ Time: ____ Initial: ____ | Date: ____ Time: ____ Initial: ____ | Date: ____ Time: ____ Initial: ____ | Date: ____ Time: ____ Initial: ____ | Date: ____ Time: ____ Initial: ____ | Date: ____ Time: ____ Initial: ____ | Comments/Conditions |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Readings |

Safety/Air Quality Monitoring

| Location: | Date: ____ Time: ____ Initial: ____ | Date: ____ Time: ____ Initial: ____ | Date: ____ Time: ____ Initial ____ | Date: ____ Time: ____ Initial ____ | Date: ____ Time: ____ Initial: ____ | Date: ____ Time: ____ Initial: ____ | Date: ____ Time: ____ Initial ____ | Date: ____ Time: ____ Initial: ____ | Comments/Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Readings |

|  |  |  |  |  |  |  |  |  |  |  |  | Appendix 3: FMP SITE DRAWING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |